|  | Simulation of Thigh540540_Aluminium Date: 6 September 2019 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Contact Information 5 Mesh information 6 Sensor Details 7 Resultant Forces 7 Beams 7 Study Results 8 Conclusion 10. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Thigh540540_Aluminium Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Fillet2; Solid Body; Mass:0.0918867 kg Volume:3.40321e-005 m^3 Density:2700 kg/m^3 Weight:0.900489 N; E:\dTrans\Rev_humanoidDesign\Mod_humanoidDesign\final\Thigh540540_Aluminium.SLDPRT Sep 06 09:23:12 2019.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (E:\dTrans\Rev_humanoidDesign\Mod_humanoidDesign\final).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-O (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 6.20528e+007 N/m^2. Tensile strength:; 1.25e+008 N/m^2. Elastic modulus:; 6.9e+010 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2700 kg/m^3. Shear modulus:; 2.6e+010 N/m^2. Thermal expansion coefficient:; 2.4e-005 /Kelvin.; SolidBody 1(Fillet2)(Thigh540540_Aluminium). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 12 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.00878525; -233.364; 0.0480728; 233.364. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 12 face(s). Type:; Apply normal force. Value:; 1000 N..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 3.24174 mm. Tolerance; 0.162087 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 16625. Total Elements; 8284. Maximum Aspect Ratio; 11.243. % of elements with Aspect Ratio < 3; 97.3. % of elements with Aspect Ratio > 10; 0.0724. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.00878525; -233.364; 0.0480728; 233.364. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 112208 N/m^2 Node: 131; 2.60977e+008 N/m^2 Node: 16106. Thigh540540_Aluminium-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0 mm Node: 57; 2.26677 mm Node: 11924. Thigh540540_Aluminium-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 2.71239e-006 Element: 8090; 0.00259391 Element: 537. Thigh540540_Aluminium-Static 1-Strain-Strain1. Name; Type. Displacement1{1}; Deformed shape. Thigh540540_Aluminium-Static 1-Displacement-Displacement1{1}.

# Conclusion